Commands:

France Router:

Int se0/0/0

En

Conf t

hostname France

Int se0/0/0

ip address 13.91.6.138 255.255.255.252

clock rate 128000

no sh

Int g0/0

Int g0/0

ip address 13.91.4.1 255.255.255.0

No sh

int se0/0/1

ip address 13.91.6.142 255.255.255.252

no sh

int g0/1

ip address 13.91.6.131 255.255.255.248

No sh

Email Server Fra:

Default gateway 13.91.4.1

Ipv4 13.91.4.2

France Router as DHCP:

ip dhcp excluded-address 13.91.4.0 13.91.4.10(excluded first 10 ip address for france)

ip dhcp excluded-address 13.91.6.0 13.91.6.10(excluded first 10 ip address for spain)

ip dhcp excluded-address 13.91.0.0 13.91.0.10(excluded first 10 ip address for portugal)

ip dhcp excluded-address 13.91.5.0 13.91.5.10(excluded first 10 ip address for germany)

ip dhcp pool spain

network 13.91.6.0 255.255.255.128

ip dhcp pool portugal

dns-server 13.91.3.5

default-router 13.91.0.1

network 13.91.0.0 255.255.254.0

ip dhcp pool germany

dns-server 13.91.3.5

default-router 13.91.5.1

network 13.91.5.0 255.255.255.0

ip helper-address 13.91.6.131(making spain a dhcp relay agent)

ip helper-address 13.91.6.138(making portugal a dhcp relay agent)

ip helper-address 13.91.6.142(making germany a dhcp relay agent)

ip dhcp pool Germany_pool

network 13.91.4.0 255.255.255.0

default-router 13.91.4.1

dns-server 13.91.3.5 (Central DNS server)

default-router 13.91.6.1(spain’s lan default gateway)

Dynamic routing protocol on France:

France(config)#router rip

France(config-router)#ver

France(config-router)#version 2

France(config-router)#no aut

France(config-router)#no auto-summary

France(config-router)#network 13.91.4.0

France(config-router)#network 13.91.6.128

France(config-router)#network 13.91.6.140

France(config-router)#network 13.91.6.136

passive-interface g0/0

Static Routing on France:

ip route 13.91.3.0 255.255.255.0 13.91.6.130

ip route 13.91.2.0 255.255.255.0 13.91.6.30

Alternate path to portugal:

ip route 13.91.0.0 255.255.254.0 g0/1 10

Portugal Router:

Int g0/0

En

Conf t

Int g0/0

ip add 13.91.0.1 255.255.254.0

no sh

Int se0/0/0

ip address 13.91.6.137 255.255.255.252

No sh

int se0/0/1

Int se0/0/1

ip address 13.91.6.145 255.255.255.252

clock rate 128000

No sh

Email Server Portugal:

Default gateway 13.91.0.1

Ipv4 13.91.0.2

Alternative path:

ip route 13.91.4.0 255.255.255.0 se0/0/1 10

Dynamic  routing:

portugal(config)#router rip

portugal(config-router)#ver

portugal(config-router)#version 2

portugal(config-router)#no auto

portugal(config-router)#no auto-summary

portugal(config-router)#ne

portugal(config-router)#network 13.91.0.0

portugal(config-router)#ne

portugal(config-router)#network 13.91.6.136

portugal(config-router)#network 13.91.6.144

Passive interface g0/0

Dhcp relay:

portugal(config)#int g0/0

portugal(config-if)#ip he

portugal(config-if)#ip help

portugal(config-if)#ip helper-address 13.91.4.1

Static Routing on Portugal Router:

ip route 13.91.3.0 255.255.255.0 13.91.6.136

ip route 13.91.4.0 255.255.255.0 13.91.6.138

ip route 13.91.6.0 255.255.255.128 13.91.6.146

ip route 13.91.5.0 255.255.255.0 13.91.6.146

portugal(config)# ip route 13.91.2.0 255.255.255.0 13.91.6.146

portugal(config)#no ip route 13.91.2.0 255.255.255.0 13.91.6.146

portugal(config)# ip route 13.91.2.0 255.255.255.0 13.91.6.138

portugal(config)#no ip route 13.91.2.0 255.255.255.0 13.91.6.138

portugal(config)#ip route 13.91.2.0 255.255.255.0 13.91.6.146

portugal(config)#ip route 13.91.6.148 255.255.255.252 13.91.6.146

portugal(config)#ip route 13.91.6.140 255.255.255.252 se0/0/0

%Default route without gateway, if not a point-to-point interface, may impact performance

portugal(config)#ip route 13.91.6.152 255.255.255.252 13.91.6.146

Spain Router:

Int se0/0/1

int se0/0/1

ip address 13.91.6.146 255.255.255.252

No sh

Int se0/0/0

ip address 13.91.6.149 255.255.255.252

clock rate 128000

no sh

int g0/1

ip address 13.91.6.129 255.255.255.248

no sh

int g0/0

ip address 13.91.6.1 255.255.255.128

no sh

Mail server spn:

Default gateway 13.91.6.1

Ipv4 13.91.6.2

DHCP Relay:

int g0/0

ip helper-address 13.91.4.1(France router lan interface address)

Dynamic routing:

Spain(config)#router rip

Spain(config-router)#

Spain(config-router)#ver

Spain(config-router)#version 2

Spain(config-router)#no aut

Spain(config-router)#no auto-summary

Spain(config-router)#net

Spain(config-router)#network 13.91.6.0

Spain(config-router)#pas

Spain(config-router)#passive-interface g0/0

Spain(config-router)#network 13.91.6.128

Spain(config-router)#network 13.91.6.148

Spain(config-router)#network 13.91.6.144

Static routing:

ip route 13.91.3.0 255.255.255.0 g0/1

ip route 13.91.0.0 255.255.254.0 13.91.6.145

ip route 13.91.2.0 255.255.255.0 13.91.6.146

Germany Router:

int se0/0/0

ip address 13.91.6.150 255.255.255.252

No sh

int se0/0/1

ip address 13.91.6.141 255.255.255.252

clock rate 128000

no sh

int g0/0

ip address 13.91.5.1 255.255.255.0

no sh

Dynamic routing protocol:

Germany(config)#router rip

Germany(config-router)#version 2

Germany(config-router)#no auto su

Germany(config-router)#no auto summ

Germany(config-router)#no auto

Germany(config-router)#no auto-summary

Germany(config-router)#ne

Germany(config-router)#network 13.91.5.0

Germany(config-router)#net

Germany(config-router)#network 13.91.6.148

Germany(config-router)#network 13.91.6.140

Germany(config-router)#pas

Germany(config-router)#passive-interface g0/0

DHCP relay:

Germany(config)#int g0/0

Germany(config-if)#ip hel

Germany(config-if)#ip help

Germany(config-if)#ip helper-address 13.91.4.1

Static routing on germany:

ip route 13.91.3.0 255.255.255.0 13.91.6.149

ip route 13.91.0.0 255.255.254.0 13.91.6.149

ip route 13.91.2.0 255.255.255.0 13.91.6.142

Argentina Router:

int se0/0/0

ip address 13.91.6.153 255.255.255.252

no sh

int g0/0

ip address 13.91.6.130 255.255.255.248

no sh

int g0/1

ip address 13.91.3.1 255.255.255.0

no sh

Email server:

Default gateway 13.91.3.1

Ipv4 13.91.3.2

Web Server:

Default gateway 13.91.3.1

Ipv4 13.91.3.3

Printer:

Default gateway 13.91.3.1

Ipv4 13.91.3.4

Central DNS Server:

Default gateway 13.91.3.1

Ipv4 13.91.3.5

DHCP Server ARG:

Default gateway 13.91.3.1

Ipv4 13.91.3.6

Static routing:

ip route 13.91.6.0 255.255.255.128 g0/0

ip route 13.91.4.0 255.255.255.0 g0/0

ip route 13.91.2.0 255.255.255.0 se0/0/0

ip route 13.91.6.148 255.255.255.252 13.91.6.129

ip route 13.91.6.144 255.255.255.252 13.91.6.129

ip route 13.91.6.140 255.255.255.252 13.91.6.131

ip route 13.91.6.136 255.255.255.252 13.91.6.131

ip route 13.91.0.0 255.255.255.254 13.91.6.131

ip route 13.91.5.0 255.255.255.0 13.91.6.131

Brazil Router:

int se0/0/0

ip address 13.91.6.154 255.255.255.252

clock rate 128000

No sh

int g0/0

ip address 13.91.2.1 255.255.255.0

no sh

Static routing on brazil:

ip route 13.91.3.0 255.255.255.0 se0/0/0

p route 13.91.6.128 255.255.255.248 13.91.6.153

ip route 13.91.6.0 255.255.255.128 13.91.6.153

ip route 13.91.6.148 255.255.255.252 13.91.6.153

Brazil(config)#ip route 13.91.6.144 255.255.255.252 13.91.6.153

Brazil(config)#ip route 13.91.6.140 255.255.255.252 13.91.6.153

Brazil(config)#ip route 13.91.6.136 255.255.255.252 13.91.6.153

Brazil(config)#ip route 13.91.4.0 255.255.255.0 13.91.6.153

Brazil(config)#ip route 13.91.0.0 255.255.254.0 13.91.6.153

Brazil(config)#ip route 13.91.5.0 255.255.255.0 13.91.6.153

default route:

ip route 0.0.0.0 0.0.0.0 13.91.6.153